# Radio Fox

Using TTGO T1 (T-Display)

# Introduction

## Radio Fox History

There is a hobby among amateur radio enthusiasts (AKA Hams) that is known as fox hunting or sometimes as bunny hunting. It is played by hiding a transmitter that transmits periodically and is then silent until it is time to transmit again. This cycle repeats. Teams use radio receivers with directional antennas to find the transmitter. The technique is called triangulation. The first team to find the transmitter wins. Sometimes the transmitter is moved periodically to make the game more challenging, this is called a walking fox.

Many of these radios are homemade using handheld radios and timers. This device is a single PCB (printed circuit board) using an esp32 (Arduino) computer with a display. There are two 18650 batteries with a charging circuit and an SA818 shortwave transceiver module on the PCB. A rotary push button dial is used to set the various parameters for the fox. These parameters include transmit time, pause time, frequency settings, music, morse code ID, various other radio settings, and a variety of system settings for the fox device.

The Bionics company manufactures several types of Fox radios. I had never heard of them until I had this project working, so I can’t claim to be inspired by any of their designs. Sven Schumacher (VE6IDK) proposed this project to me in 2023, at which time I suggested using the TTGO T-Display esp32 module. I had used this device in a previous project, my Magic Image Wand, so I already had a nice menu system that would be easy to adapt to this project. It also featured code to handle a micro-SD card which could be used to store configuration files and music/tone files.

A link to this file can be found here: https://www.youtube.com/watch?v=oTXZL2KgMQE

## Antenna

Any 50-ohm antenna designed for the VHF range is acceptable. Please remember to connect the antenna before transmitting, it is safest to have the proper load connected to the radio chip. You can also connect a 50-ohm dummy load for testing purposes to avoid sending radio waves out into the air!

## Initial Setup

The first thing to enter is the Call Sign. Remember that you should not be transmitting without sending your call sign out periodically. The radio cycles between Call Sign and Beacon (whatever text you want) and the music if a music file has been selected. The Beacon and Call Sign are sent out as morse code. A future revision of the PCB is being considered that will have an audio chip so sound recordings can also be sent, but for now just morse code and music tones. The speed and pitch of the morse code can be adjusted.

The Beacon text is often something that will identify this radio, especially if there are more fox radios in the same neighborhood.

## Charging Issues

The device is charged using the USB-C connector. There is a flaw on the PCB that requires a USB-A to USB-C cable. This has been fixed in version 1.11 of the PCB so that USB-C to USB-C cables will also work.

## Beacon Mode

Version 1.06 adds a beacon mode that limits the allowed frequency range. The system must be powered by an external power supply connected to the USB C charging port if you want it to keep running for days, weeks or longer. This Beacon mode should not be confused with the Beacon text that is transmitted periodically.

## Source Code

The firmware source code consists of C and C++. The source code can be found at MartinNohr/RadioFox: Ham Radio Fox Software

### Arduino Library Code Versions

Typically, the latest version of libraries is used. The actual versions will be listed here when the source code is released.

This is the software version of the custom code. The changes are described below.

Version 1.14

- Added ability to play WAV and MP3 files. Removed the immediate txstop option as it could leave the audio player in an untidy state. Updated some libraries to the latest versions.

Version 1.13

- Ported the code to espressif core 3.x instead of requiring 2.0.17.

Version 1.12

- Moved latitude and longitude menu items to be right below the “Send GPS:” menu item. They were below the “TX:” menu which was confusing. The TX frequency item was moved right below the TX Power and before the beacon mode item.

- I also corrected the documentation that the power output is 1 or 0.5 Watts for high and low power settings.

Version 1.11

- Added ability to send manually entered GPS coordinates when in beacon mode.

Version 1.10

- Added main menu entries to return to the main run screen. The long press still works.

Version 1.09

- Moved the active time display to the end of the second line on the main display. Bug 32, when the audio file was selected, the last line overwrote the bottom of the display.

Version 1.08

- Show the active run time on the main display in hours, minutes, and seconds. This time is reset on boot, and just keeps track of the time used for sending and pausing.

Version 1.07

- Change "Beacon Frequency Mode" in menus to "Beacon Frequency".

Version 1.06

- Added a switch to limit the frequency range to that allowed for beacon use in ITU region 2. The range is 144.276-144.300 MHz.

Version 1.05

- Added adjustable morse pitch, from 300 to 3000 Hz.

Version 1.04

- Fixed crash when no SD card found.

Version 1.03

- Fixed hang problem #26. Short TX time and B0 pressed caused the display to stop updating.

Version 1.02

- Changed prompts from BTN0/1 to B0/1 to match the labels on the case.

Version 1.01

- Made TX red dot larger. Show DTMF characters in green on the bottom. (not tested yet).

Version 1.00

- This is initial version when this document was first created. The software is considered mostly complete.

# The Hardware

## PCB

The Fox is a single PCB that includes an 18650 battery holder that can have 1 or 2 batteries. The button top batteries are used. The flat top batteries will not work. Also note that the size standard is not always followed and some batteries will not fit. If that is the case try a different pair. The best batteries I have found are from www.liionwholesale.com. The batteries I use are found here: Protected 3400mAh 10A 18650 Button Top Battery - Wholesale Discount – Liion Wholesale Batteries .

There is a USB C connector on the left edge used for charging the batteries. Above that is a micro-SD slot. The SD card holds tone files and saved settings files. It can also be used to load new firmware onto the TTGO chip. There is also a USB C connector on the TTGO. It is used for downloading firmware or debugging operations. It might also be used for serial communications in a future firmware release. It is not used for charging the batteries.

## Radio

The radio functions are handled by the SA818-V on the top right. There are vents in the case to allow cooling as this module can become quite hot when transmitting at high power for longer timer intervals. Transmitting longer than one minute is not recommended. High power is about 1W and low power is about 0.5W. These powers are according to the data sheet and may not be entirely accurate. I have not measured them with a power meter. Make sure a suitable antenna is connected to avoid damage to the radio circuit.

## Batteries

The 18650 batteries must be button top style and protection is recommended. The 18650 battery length is a very loose standard, and some batteries may not fit. Too long and they cannot be used. Too short, it might be possible to bend the contacts on the battery holder negative side. The battery holder is polarized to prevent insertion in the wrong direction. Do not defeat the feature or magic blue smoke may appear from the PCB!

# Run Time

The actual runtime depends on the transmitting time and quality of the batteries. There are many deceptively labelled 18650 batteries available. The best one are made by Panasonic and Sanyo. Using 1 minute transmit, 4 minutes pause, low power setting, and display dimming the Fox Radio ran for 12 hours using Panasonic batteries.

# Tone Files

The tone files are used to play “music” during transmissions. This can be used to identify the Fox, or just to annoy people!

The files are saved as .TXT and simply contain a list of tones to be played. The format is similar the hard coded files from here: robsoncouto/arduino-songs (github.com)

The Robson GitHub site has many other example tone files. They are arrays in code but are easily converted to the required text format. Give the file a name and save it on the SD card.

## Example Tone File

This is the Tetris theme music

there are two values per note (pitch and duration)

make sure each token ends with a ','

and that the data starts with the tilde which must not be in these comments

LENGTH, %val can be used to set the length of the note, it defaults to 90% and 10% quiet after the note

This can be used for pizzicato notes, LENGTH, 25, for example

~

TEMPO, 145,

LENGTH, 90,

NOTE_E5, 4,  NOTE_B4, 8,  NOTE_C5, 8,  NOTE_D5, 4,  NOTE_C5, 8,  NOTE_B4, 8,

NOTE_A4, 4,  NOTE_A4, 8,  NOTE_C5, 8,  NOTE_E5, 4,  NOTE_D5, 8,  NOTE_C5, 8,

NOTE_B4, -4, NOTE_C5, 8,  NOTE_D5, 4,  NOTE_E5, 4,

NOTE_C5, 4,  NOTE_A4, 4,  NOTE_A4, 8,  NOTE_A4, 4,  NOTE_B4, 8,  NOTE_C5, 8,

NOTE_D5, -4, NOTE_F5, 8,  NOTE_A5, 4,  NOTE_G5, 8,  NOTE_F5, 8,

NOTE_E5, -4, NOTE_C5, 8,  NOTE_E5, 4,  NOTE_D5, 8,  NOTE_C5, 8,

NOTE_B4, 4,  NOTE_B4, 8,  NOTE_C5, 8,  NOTE_D5, 4,  NOTE_E5, 4,

NOTE_C5, 4,  NOTE_A4, 4,  NOTE_A4, 4, REST, 4,

# Audio Files

It is possible to play MP3 or WAV files instead of the tone files. They are selected through the same menu selection. The radio uses PDM (pulse density modulation) to encode the audio. This is not the best audio quality, but it does allow the use of the existing board design. Better quality would require a proper sound chip on the PCB. This may be done in the future.

# Updating Firmware

The new file must be put on the SD card and named RadioFox.bin. The system menu has an entry New Version BIN file that will attempt to load this file. Confirmation will be required from the dial, After loading the BIN file can optionally be erased.

# DTMF Operations

DTMF (aka touch-tones) can be sent from the transmitter to control certain operations of the radio. It works best when the tones are sent for at least a second. It is usually also necessary to wait 2 seconds after pressing the transmit button on the radio. All commands consist of a ‘*’ followed by a digit. The following commands are available.

Please feel free to request additional commands. I’m sure there are other usual things that can be done remotely using DTMF signals. Also note that these functions have not yet been tested extensively so there may be some bugs in the code. I will fix as soon as I can after I find out what they are.

## DTMF *1

Start transmitting timer.

## DTMF *2

Stop transmitting timer.

## DTMF *4

Set low power mode.

## DTMF *5

Set high power mode.

## DTMF *6

Cancel existing delay and pause and begin transmitting immediately.

# Button Operations

All operations can be controlled by the rotary push button on the box and the two extra buttons to the left labelled B0 and B1. A long press is used to access the menu system and is done by holding the button down for about a ½ second or longer. This long press time is also adjustable if a longer or shorter time is preferred. The system has two modes of operation. After booting the display shows the radio status and some important radio settings.

A long press on the dial or a simple press on B1 will open the menus. The system opens the last menu that was used. A long press or B1 in the menus returns to the normal radio mode. Each menu except the main menu has two entries, the first and the last, which will return to the previous menu.

Many data entry screens will show help on the bottom few lines to explain the button functions.

## Dedicated Buttons – Button0(B0) and Button1(B1)

There are two dedicated buttons on the top left of the TTGO device. They are accessible from the top of the case. The top one is called B0 and the bottom one is called B1. Both these buttons support both clicking and long presses. A long press of B0 and B1 together will reboot the system. The function of these buttons depends on the current operating mode as described next. Common to all modes is that clicking B1 will always emulate a long press of the dial button.

### B0/B1 During Run Mode

B0 toggles the run state to stop or start transmitting mode. Long B0 cancels the current transmit or pause timer. B1 enters the menu system.

### B0/B1 During Menu Mode

B0 returns to the previous menu. It does nothing if at the top menu. Long B0 displays some help (if available) about the current menu item. Note that currently only a few menu items have help. More will be added in the future.

B1 acts the same as long pressing the dial button.

### B0/B1 During Integer Entry

B0 cycles through the allowed increment values. For example, if the range is 1 to 1000, the increment from the dial will be 1. After a dial click the increment will change to 10 and dial will add and subtract values by 10. Another click will change the increment to 100. Long B0 resets the value to the original value. B1 or long dial click saves the new value and closes the number entry screen.

## Rotating Dial and Clicking Button

Dial rotation is only useful in the menu system, it doesn’t have any function while the radio status is displayed. In menu mode the dial menu selects items, and changes settings. In the menus, boolean values toggle with each click. Multiple selections cycle through all the available options. Integer values go to another screen that shows the minimum and maximum values. Rotating the dial right increases the value while rotating to the left decreases it by the increment amount. The increment amount can be changed by clicking the button which will cycle between different values. One of the values will be a reset to the original value, activated with a long press. A long press is also used to accept the new value and return to the menu. A * or highlight indicates the active menu line. A + in front of the menu means another menu will be activated. Any menu item starting with ‘-‘ goes back to the previous menu when clicked. A dial long press will return to radio status mode. The current menu entry is remembered so that it will return to the same place when the long press is done from the radio status mode.

### Button Menu Operations Short Descriptions

Left/ Right

Select menu

Select file or list values

Inc/ Dec integers

Long press

Open menu system, goes to the remembered menu point

Return to radio mode, the current menu point is remembered

Accept integer

Choose filenames

Click

Select menu item, including return to previous menu

Toggle booleans

Cycle through multiple selections

Cancel filename or list selections

# Main Operating Screen

After booting or when not in the menu system the radio status is displayed. It shows the current operation, the count of transmit cycles, the ID text, any selected audio file, the transmit frequency and the receive frequency offset. The bottom of the screen shows the battery status on the right if it has been enabled. This is related to the current battery voltage and will drop while transmitting. There are also 4 LED’s on the PCB that can be seen through the slot on the side of the case. These LED’s are controlled by the battery charger circuit and indicate the battery status during running and charging.

During transmitting the bottom left shows an animated red dot. In a future firmware update the received DTMF characters will be display next to this.

# Menu Details

The following sections detail each entry in all of the menus starting with the main or top menu. A ‘+’ at the start indicates that this is a sub-menu and will open more menu selections when clicked. Menu items that have a ‘:’ after the name will show options separated by ‘|’ or a ‘-‘ to indicate a range. Values in italic are alphanumeric sequences. An example is the radio ID. Some menu entries may depend on other menu items settings. For example the audio file name menu item is hidden unless “Play Audio File” is Yes.

## +Radio Settings

### XMIT: On|Off

This must be set to On for the radio to begin transmitting. If saved as On then the radio will start sending right after booting. If it is Off it will not automatically start sending. It can be toggled with the B0 button while in run mode. Using B0 does not changed

### TX Power: High|Low

High power is about 1W and low power is 0.5W. These are the only values allowed by the SA818 radio. If lower power is desired an attenuator can be attached to the radio antenna.

### TX: 134-174 MHz or 144.275-144.300 (beacon mode only)

This is the transmit VHF frequency. A future version might be created using the UHF radio module. VHF is more common for Fox Hunting.

### Beacon Mode: Yes|No

This restricts the frequency range to the allowed values for ITU region 2.

### Send GPS: Yes|NO (beacon mode only)

This enables sending the GPS latitude and longitude values. The values must be entered manually from a phone or GPS device. A future version of the hardware may have a built-in GPS module or it may be able to connect to a serial GPS device using the USB connection.

### Latitude: +/-n.nnnnnn (Send GPS Yes only)

Enter the current latitude by reading it from a phone or GPS unit.

### Longitude: +/-n.nnnnnn (Send GPS Yes only)

Enter the current longitude by reading it from a phone or GPS unit.

### Beacon ID: text

This is an alphanumeric string sent out during the transmitting cycle as morse code. It is always sent right before the Call Sign (ID). The beacon text may be empty.

### Call Sign: text

This alphanumeric string is sent out after the beacon text. It is sent as morse code. It is legally required to send this ID string periodically so it must not be empty and should be a valid FCC amateur radio ID. It is usually set to the ID of the person setting up the radio.

### Play Audio File: Yes|No

This allows the selected tone file to be played after the beacon and call sign are sent.

### Audio File: filename

This menu item is only visible if the previous item is set to Yes. Clicking the dial will list all of the *.txt files on the SD card. The dial is rotated to highlight a file and a long press will set it. Note that a click of dial just exits the list of file with no selection being performed.

### Morse Interval: 50-500 mS

This sets the speed of the morse code sent using the Beacon and Call Sign values. Lower values are faster, larger values are slower.

## +Radio Timers

### Start Delay: 0-120 Min

When B0 or the menu starts the transmit mode the system will not actually begin sending until this time is expired. This allows the radio to be hidden and gives the hiding person time to leave the area and return before transmission starts.

### Timer Mode: Fixed|Random

In Fixed mode the sending time and pause timer are controlled by the Send and Pause values and are always the same. In Random mode the transmit time and pause time are randomly chosen from the allowed values set in the Min and Max values for TX Send and Pause. This makes the timing less predictable and make the game a little bit harder. Keep your ears on!

### TX Send: 10-90 Sec

Fixed time for transmission. Shorter times are more challenging to locate but use less battery energy. Longer times overheat the radio and use more battery energy. If this time is set to less than the total ID+Call+Audio time then there will be only one cycle of data transmitted. If the time is longer then the ID+Call+Audio cycles will be repeated until the time has expired. Note that both ID and Audio will always be completed before transmission ends.

### TX Pause:10-1200 Sec

This is the pause between transmissions. Longer times let the radio cool off and save the battery for longer run times.

### TX Send Min: 10-90 Sec

In random mode this is the minimum send time.

### TX Send Max: 10-90 Sec

In random mode this is the maximum send time.

### TX Pause Min: 10-1200 Sec

In random mode this is the minimum pause time.

### TX Pause Max: 10-1200 Sec

In random mode this is the maximum pause time.

### Radio Pause: Sleep|Awake

During pause time the radio can be put into sleep mode. This lowers the power consumption and lets the battery last longer. However note that this also means that the DTMF commands are disabled since the radio is asleep!

## +More Radio Settings

### RX Offset: -600|0|+600 kHz

This is the frequency offset from the transmit frequency that the radio uses to receive DTMF commands. An offset is recommended. If 0 is used the radio will only listen when it isn’t sending. It has not been tested if it listens during sending when the offset is not 0.

### Bandwidth: 12.5|25 kHz

This sets the radio bandwidth. It has not been tested much to see what effect it has on performance yet.

### CTCSS/DCS: CTCSS|DCS

This sets the un-squelch values for either the analog or digital versions.

### TX CTCSS: None|67-250.3

Analog un-squelch frequencies sent out.

### RX CTCSS: None|67-250.3

Analog un-squelch frequency for receiving.

### TX DCS: None|023-754

Digital un-squelch value for sending.

### TX I/N: I|N

The digital I or N value for sending.

### RX DCS: None|023-754

Digital un-squelch value for receiving.

### RX I/N: I|N

The digital I or N value for receiving.

### RX Volume: 1-6

This is the receive volume setting. It affects how well DTMF tones are detected.

### RX Squelch: 0-8

This affects how much signal is required to receive the DTMF signals.

## Cancel Waits/TX Now

When the radio is in either send or pause mode this command will cancel the timer and the next mode will start. For example, if you want to cancel the pause and go to transmit, use this option. It can also be done by long pressing B0.

## +Saved Settings Files

Saved settings files are named files stored on the SD card. They are useful to quickly configure the radio for different uses. All values except the battery calibration are affected.

### Create File

This prompts the user for a name. The file is then created using the current system settings.

### Save/Update File

This command saves the current settings into the selected file. If it already exists the user will be asked to confirm overwriting the file.

### Load File

This command asks the user for the name and then sets all of the system settings except the battery calibration using the values in the file.

### Delete File

The user selects a file from the list and then it can be deleted.

## +System Settings

### +Display Settings

#### Dimming Mode: None|Timer

The display can be set to dim automatically after a specified number of seconds. This is used to save battery power since the backlight uses a significant amount.

#### Bright Value: 1-100%

Set the display brightness value.

#### Dim Value: 0-100%

Set the display brightness after the timer below expires.

#### Display Dim Time: 0-120 S

Specify the time until the display changes to dim mode.

#### +Sideways Scroll Settings

Lines of text that too long for the display will scroll back and forth. The following settings control the speed.

##### Speed: 1-1000 mS

Controls the speed of text moving to the left.

##### Pause: 1-100 mS

This controls the pause when the text is at the end (all the way to the left).

##### Reverse Speed: 1-20x

This is how fast the text scrolls back to the beginning (to the right). 2x means twice as fast as the text scrolled to the left.

#### Menu Selection: Color|*

Menu selections and selections from a list can be either colored or marked with the ‘*’ character.

#### Text/HiLite Colors

This selection allows for choosing the colors of the text and the highlight. The background screen is always black.

### +Dial & Button Settings

#### Direction: Normal:Reverse

If you really want the button to be the other way around this is the choice for you.

#### Long Press Timer: 2-200

This value affects how long or short a long press is. The default value is normally OK, but somebody might want to change it.

### +Battery Settings

Since the PCB was designed with 10% resistors for measuring the battery voltage some calibration is needed. A future PCB will probably use better resistors so this won’t be needed.

#### Show Battery: Yes|No

This option controls the battery display on the bottom right of the display.

#### Full Battery: 2000-4000

This is the value that indicates a full 4.2V battery.

#### Set Full Battery

This selection reads the current battery values and stores it. It is best set by fully charging the battery and then disconnecting the charger. Wait a couple of minutes and then use it several times until it seems stable.

#### Read Battery Raw Data

This continuously reads the battery value. It was originally used for testing, but it seems like it might possibly be useful so it was left in the menu system.

### New Version BIN File

New firmware can be loaded by putting the binary file on the SD card. It must be named RadioFox.BIN. There are two prompts, one to confirm overwriting the firmware, and another to allow deletion of the BIN file after installation.

### Factory Settings

This command resets all settings except the high battery value to the original factory values. The factory reset can also be done by holding the dial button down while turning the power on.

### Format EEPROM

This will clear and reset the eeprom where the settings are save in non-volatile memory. This is how settings are saved while the system is not powered.

## Save Current Settings

This stores all of the current settings in the non-volatile memory. Note that this is not often used since settings are normally saved each time they are changed.

## Reboot

This causes the system to reboot. It will wait 2 seconds first. If you press the dial during the reboot factory settings will be restored. Note that the battery full settings will not be reset.